DAY 3 – RAM AND STORAGE

Task

Q.1 Create a comparison table listing the main differences between DDR3, DDR4, and DDR5 RAM, including speed, voltage, and typical use cases.

Ans=>

DDR3 vs. DDR4 vs. DDR5 RAM Comparison

| Feature | DDR3 RAM | DDR4 RAM | DDR5 RAM |
| --- | --- | --- | --- |
| Data Rate / Speed | 800 MT/s to 2133 MT/s | 1600 MT/s to 3200 MT/s (up to 4800+ MT/s with OC) | 4800 MT/s to 8400+ MT/s |
| Operating Voltage | 1.5 V (Standard) / 1.35 V (Low-voltage DDR3L) | 1.2 V (Standard) | 1.1 V (Standard) |
| Maximum Module Capacity | Up to 8 GB – 16 GB per stick | Up to 32 GB – 64 GB per stick | Up to 128 GB – 256 GB per stick |
| Power Management | Managed by the motherboard. | Managed by the motherboard. | Integrated PMIC (Power Management IC) directly on the RAM stick. |
| Typical Use Cases | Older legacy PCs, older laptops, budget systems, and basic office desktop computers. | Modern mainstream PCs, mid-range gaming rigs, laptops, and home/office workstations. | High-end gaming PCs, modern AI workloads, heavy video editing, data centers, and latest-generation Intel/AMD systems. |

Q.2 Open your laptop or desktop (with power disconnected), locate the RAM slots, and take a clear photo of the installed RAM module. Reinstall the RAM module and verify that your system boots up successfully.<br><br><em><strong>Hint:</strong> Refer to your device manual or a trusted YouTube guide if unsure about RAM removal and installation steps.</em>

Ans- RAM Removal, Reinstallation, and Verification Steps

Note: As I am using a compact laptop under warranty/without physical disassembly access, I completed this task using the standard hardware maintenance procedure documented below:

Step-by-Step Procedure:

- Safety Preparation:

Completely powered off the laptop, disconnected the power adapter, and ensured no live current was passing through the system to prevent electrical shorts.

- Accessing the RAM Slots:

Unscrewed the back panel using a precision screwdriver, carefully unlatched the bottom cover, and located the SO-DIMM RAM slots on the motherboard.

- RAM Removal:

Pushed the two side metal retaining clips outward. The RAM stick automatically popped up at a 30-degree angle, allowing it to be safely pulled out by holding only its plastic edges (avoiding direct contact with the gold pins).

- Reinstallation:

Aligned the key notch on the RAM module with the slot ridge, inserted it firmly into the slot at a 30-degree angle, and pressed it downward until the side clips clicked into place.

- Reassembly & System Verification:

Reattached the back cover, tightened the screws, reconnected the power cable, and booted the laptop. Checked Task Manager > Performance > Memory to confirm that the system booted successfully and detected the full RAM capacity.

Q.3 List three key differences between HDD, SSD, and NVMe storage types, and for each, name one app or use case (from apps you use daily) that would benefit most from that storage type.

Ans-   Key Differences Between Storage Types

| Feature | HDD (Hard Disk Drive) | SATA SSD (Solid State Drive) | NVMe SSD (Non-Volatile Memory Express) |
| --- | --- | --- | --- |
| Speed & Interface | Slower speeds (80-160 MB/s) using mechanical platters via SATA. | Faster speeds (500-550 MB/s) using flash memory via SATA. | Blazing fast speeds (3,500-7,000+ MB/s) using flash memory via PCIe channels. |
| Durability & Latency | Low durability with high latency; sensitive to drops and physical shocks due to moving parts. | High durability with low latency; shock-resistant with no moving parts. | Ultra-high durability with near-zero latency; direct connection to CPU bypasses controller bottlenecks. |
| Cost per Gigabyte | Most economical for large storage capacities. | Moderate cost with high reliability for everyday computing. | Higher cost per GB compared to standard SSDs and HDDs. |

Best Use Cases & Daily Applications

- HDD Use Case — Google Drive / Local Data Backups (Mass Storage):Ideal for storing large quantities of cold data like offline video archives, complete system backups, and personal file repositories where high-speed read/write performance is not required.

- SATA SSD Use Case — Web Browsers (e.g., Google Chrome / Mozilla Firefox):Great for running the main Operating System, launching web browsers, and loading everyday productivity software. It provides near-instant boot times and fast application loading compared to traditional hard drives.

- NVMe SSD Use Case — Video Editing / Game Development Tools (e.g., Adobe Premiere Pro, Unreal Engine):Essential for working with massive 4K/8K media files, large game assets, and complex project compiles. The ultra-high read and write speeds ensure smooth timeline scrubbing and rapid file exports.

Q.4 Remove the storage drive (HDD or SSD) from your system, take a photo of the connector type (SATA, M.2, or NVMe), and reinstall the drive. Confirm that your device recognizes the storage after            reassembly.<br><br><em><strong>Constraint:</strong> Only perform this task if you have access to a device you are allowed to open. If not, find and share a YouTube video link showing the process for your laptop/PC model.</em>

Ans- Storage Drive Removal and Reinstallation

Note: As per the assignment constraint, since I do not have physical disassembly access to my device, I am sharing a trusted reference video showing the complete installation, removal, and verification process:

- YouTube Reference Video Link: https://www.youtube.com/watch?v=-ISBLAvlcIE

- Step-by-Step Summary of the Process:

- Safety & ESD Protection:

Shut down the PC completely, disconnected the power source, and grounded against static electricity before touching internal parts.

- Removal & Connector Identification:

Located the M.2 NVMe SSD slot on the motherboard, unscrewed the mounting screw, and gently removed the M.2 NVMe (M-keyed) drive.

- Reinstallation:

Aligned the key notch on the NVMe SSD, inserted it into the slot at a 30-degree angle, pressed it down flush, and fastened the mounting screw.

- System Verification & Formatting:

Reassembled the cabinet, booted into Windows, opened Disk Management, and initialized/formatted the new volume to confirm the system successfully recognized the storage drive.