DAY 4 – GPUs AND COOLING SYSTEMS

TASKS

Q.1 Open your own laptop or desktop (or use an online PC hardware simulator if you don't have access to a real system) and identify the GPU model installed. Write down the GPU's brand, model number, and whether it is integrated or dedicated.

Ans- 1. Identification Method

- Tool Used: Windows System Information (msinfo32) & Device Manager

- Target Device: HP Laptop 15s-fq5xxx

2. GPU Hardware Specifications

- GPU Brand: Intel

- GPU Model: Intel(R) Iris(R) Xe Graphics

- GPU Type: Integrated Graphics (Embedded on the 12th Gen Intel Core i5-1235U CPU die, sharing memory dynamically with the 8.00 GB system RAM)

Q.2 Take a clear photo or screenshot of the cooling system (fan, heatsink, or liquid cooling setup) inside your PC or from a PC hardware simulator. Label the main cooling components and briefly describe their function.

Ans-

1. Cooling Setup Reference

- Laptop Model: HP Laptop 15s-fq5xxx

- Cooling Setup Type: Air Cooling System (Blower Fan + Copper Heat Pipe + Heatsink Fins)

- Reference Visual Guide: HP 15s Series Internal Cooling Fan & Heatsink Setup

2. Main Cooling Components & Functions

- 1. Copper Heat Pipe:

  - Function: Transfers thermal energy (heat) directly away from the 12th Gen Intel Core i5-1235U CPU die toward the heatsink fins using liquid-to-vapor phase change inside the sealed copper tube.

- 2. Heatsink & Cooling Fins:

  - Function: Absorbs heat from the copper heat pipe and dissipates it over a larger surface area made of thin metal fins located right next to the exhaust vent.

- 3. Centrifugal Blower Fan:

  - Function: Draws cool air from the bottom intake vents of the laptop chassis and pushes it through the heatsink fins, forcing hot air out through the rear exhaust vent.

- 4. Thermal Paste / Thermal Interface Material (TIM):

  - Function: Fills microscopic air gaps between the surface of the Intel CPU and the copper heat pipe block to ensure maximum thermal conductivity.

Q.3 Create a comparison table listing at least 3 differences between air cooling and liquid cooling systems for GPUs, including factors like cost, efficiency, and maintenance.

Ans-

| Factor | Air Cooling System | Liquid Cooling System (AIO / Custom Loop) |
| --- | --- | --- |
| Cooling Efficiency & Temps | Moderate: Relies on heatsinks and fans. Maintains safe operating temps under standard loads but thermal throttles faster under extreme overclocking. | High: Uses water/coolant to transfer heat away faster, resulting in significantly lower thermal output and consistent boost clocks. |
| Cost & Budget | Low to Moderate: Budget-friendly, lower manufacturing cost, and widely available out-of-the-box on most GPUs. | High: Considerably more expensive due to complex parts like radiators, water blocks, pumps, and tubing. |
| Maintenance & Reliability | Low Maintenance: Highly reliable with long lifespan; only requires periodic dust cleaning off fans and heatsinks. | Moderate to High: Requires checking for pump failures, liquid evaporation, or potential coolant leaks over time. |

Q.4 Imagine you are building a gaming PC for streaming IPL matches and playing graphics-heavy games. Choose between air cooling and liquid cooling for your GPU, and justify your choice in 3-4 sentences based on your research.

Ans-

Selected System: Liquid Cooling System

Justification:

For a multi-purpose gaming PC intended for streaming IPL matches while simultaneously running graphics-heavy games, Liquid Cooling is the optimal choice for the GPU. High-framerate gaming paired with real-time video encoding/streaming places a continuous, maximum thermal load on the GPU die. Liquid cooling dissipates heat much more efficiently than air cooling, preventing thermal throttling and keeping clock speeds consistently high. Additionally, liquid cooling operates at significantly lower noise levels, ensuring clean, background-noise-free audio during live streams.